Theory — LFM Chirp & Dechirp

SDR testbed pod · Topics 2 (Modulation/I-Q) & 5 (Detection)

Background

An LFM (linear-FM) chirp sweeps its frequency linearly across a bandwidth B during a sweep time T. It is the core waveform of FMCW radar and of Martin's masking/sensing work.

When the chirp reflects off a target at distance R, the echo returns delayed by tau. The dechirp trick multiplies the echo by the complex conjugate of the transmitted chirp; the quadratic phase terms cancel and leave a single constant-frequency tone — the beat frequency — proportional to the delay.

Taking the FFT of the dechirped signal shows a peak at the beat frequency, which maps directly to target range. Working in complex baseband (I/Q) is what makes this clean.

Key formulas

transmit chirp:  s(t) = exp( j*pi*(B/T)*t^2 )

beat frequency:  f_b = (B/T) * tau

dechirp:         mix = echo * conj(s);   FFT(mix) peaks at f_b

The quantum bridge

I/Q processing is exactly what a coherent (homodyne) quantum receiver does — the CV path. So this exercise is the same signal chain we reuse when we move from SDR toward quantum detection.

What you implement

A function estimate_beat_freq(fs, B, T, tau) that dechirps the echo, FFTs it, and returns the peak frequency.

How you know you got it right

For a known delay tau, your estimate should match (B/T)*tau to within a couple of FFT bins; the checker verifies several delays.

See INSTRUCTIONS.md in this folder for the step-by-step task, then run the checker.