Theory — BER of BPSK over AWGN

Simulations pod · Topic 3 (Noise, SNR & BER)

Background

A digital link carries bits by mapping them to waveforms (symbols). The simplest scheme is BPSK: bit 0 becomes -1 and bit 1 becomes +1 — two "antipodal" points as far apart as possible for a given power.

Real channels add noise. The standard model is AWGN — additive white Gaussian noise — where each sample gets an independent Gaussian perturbation. The receiver decides the bit from the sign of the noisy sample.

We measure reliability with the Bit Error Rate (BER): the fraction of bits received wrong. BER falls as the signal-to-noise ratio rises. Eb/N0 (energy per bit over noise density) is the fair way to compare systems.

Key formulas

BER_theory = 0.5 * erfc( sqrt(Eb/N0_linear) )

Eb/N0_linear = 10 ** (Eb/N0_dB / 10)

noise std for unit-energy BPSK: sigma = sqrt( 1 / (2 * Eb/N0_linear) )

The quantum bridge

BER is the direct ancestor of the quantum bit-error rate (QBER). The same measurement tells us how well an authorized receiver (Bob) does versus an eavesdropper (Eve), and the QBER threshold (~11%) decides whether a secure key exists at all.

What you implement

A function simulate_ber(Eb/N0) that modulates random bits, adds AWGN, detects, and returns the measured BER.

How you know you got it right

Your simulated points should sit on top of the theoretical curve and drop steeply as Eb/N0 increases; the checker confirms this within statistical tolerance.

See INSTRUCTIONS.md in this folder for the step-by-step task, then run the checker.