Theory — Channel Capacity & Link Loss

Simulations pod · Topic 4 (Channel: loss & capacity)

Background

Every link is bounded by two numbers. Shannon capacity is the maximum error-free rate for a given bandwidth B and signal-to-noise ratio: more bandwidth or higher SNR means more capacity.

The received SNR itself comes from a link budget: start with transmit power, subtract the path loss, subtract the noise power — all in decibels, so it is simple addition and subtraction.

Key formulas

capacity:     C = B * log2(1 + SNR_linear),   SNR_linear = 10**(SNR_dB/10)

link budget:  SNR_rx(dB) = P_tx(dBm) - path_loss(dB) - noise_power(dBm)

The quantum bridge

Quantum channels are loss-limited: the achievable secret-key rate is capped by channel transmissivity (the repeaterless PLOB bound), and Shannon capacity generalizes to secrecy capacity. The same dB reasoning applies to photonic links.

What you implement

Two functions: shannon_capacity(bandwidth_hz, snr_db) and rx_snr_db(tx_power_dbm, path_loss_db, noise_power_dbm).

How you know you got it right

The checker compares your capacity to B*log2(1+SNR) and your link budget to the dB subtraction, across several cases.

See INSTRUCTIONS.md in this folder for the step-by-step task, then run the checker.