Theory — Repetition Code (Error Correction)

Simulations / Theory pod · Topic 6 (Channel coding)

Background

Forward error correction adds structured redundancy so the receiver can fix errors without asking for a resend. The simplest example is a repetition code: send each bit several times.

Over a binary symmetric channel (each bit flips with probability p), the receiver majority-votes each group back to one bit. For three repetitions, a decoded bit is wrong only if two or three copies flip — which is less likely than a single bit flipping, so the code lowers the error rate.

Key formulas

3x majority error:  P(error) = 3*p^2*(1-p) + p^3 = 3*p^2 - 2*p^3

and for small p this is much smaller than p  (coding gain)

The quantum bridge

QKD/QSDC use error reconciliation to correct the sifted key, and quantum error-correcting codes protect qubits directly — both are quantum descendants of this idea, and coding is Michel's specialty.

What you implement

A function repetition_ber(p_flip, n_rep) that encodes, passes through the BSC, majority-decodes, and returns the decoded BER.

How you know you got it right

Your decoded BER should match 3p^2 - 2p^3 (for 3x) and be clearly lower than the raw channel error p — the checker confirms both.

See INSTRUCTIONS.md in this folder for the step-by-step task, then run the checker.